Section VI.

## Impacts on Areas of Concern

Q26. Is proposed aquaculture facility in the “zone of influence” of a designated Area of Concern (Consider also state and provincial AOC’s and Aquatic Biodiversity Investment Areas)?

Proceed to Section VII, Effects of Settleable Solids on Benthos and Shellfish

A2. no

A1. yes

Go to Question 30.

Q27.  Are there recovery plans for fish or wildlife in the Remedial Action Plan? Consult web page: www.cciw.ca/glimr/raps/aoc-map.html

A2. no

Q29. Could escaped cultured species significantly harm area species by interbreeding, predation or competition for food or habitat?

A2. no

A1. yes

Q28. Are there fish-eating predators (e.g. birds) in recovery plans?

A1. yes

A2. No. It is known that they could not adversely affect area species.

A1. yes

H15. Fish-eating predators may be at risk from harassment. If willing to accept risk, ensure protective, secure predator apparatus is included in proposal and go to Q. 29.  If unwilling to accept risk, consider relocation of facility to area not heavily populated with fish-eating predators.

H16. The recovery species in the AOC plan may be at risk from interspecific hybridization, predation or competition for food or habitat. More information is needed about the recovery species before a decision can be reached. Contact local lead person at www.cciw.ca/glimr/raps/aoc-map.html. If willing to accept risk, go to Question 30. If not, consider culturing different species or relocate to a site where adverse interactions with recovery species are less likely to occur.

Go to Question 30.

Q30. Does recovery plan include improvement of degraded benthos?

Q31. Does Recovery Plan identify eutrophication or undesirable algae as evidence of impaired beneficial use?

A2. no

A1. yes

Accept risk

H17. Benthic organisms may be at risk unless measures are in place for the removal of excess food, feces and mortalities. If these measures are in place, and you are willing to accept this risk, proceed to Question 32. If not, consider relocation of proposed facility.

H18. Recovery plans may be hampered by the addition of an aquaculture facility in this area. A mass balance analysis may be necessary to quantify risk. Contact appropriate federal, state or provincial agency for assistance. If it is determined that water quality will not be adversely affected, and you are willing to accept this risk, go to Question 32.

A2.

no

A1. yes

Q32. Does the AOC at issue identify remediation of contaminated sediments? Consult web page: www.cciw.ca/glimr/raps/aoc-map.html

accept risk

A2. no

H19. Contaminated sediments may pose a hazard  to cultured organisms due to possible  exposure to contaminants. This may also pose a hazard to food safety. If willing to accept this risk, proceed to Section VII, Effects of Settleable Solids on Benthos and Shellfish. If not, consider relocation of facility.

Proceed to Section VII, Effects of Settleable Solids on Benthos and Shellfish

A1. yes